# Monthly → Daily → Hourly Load Curve Methodology (Enhanced with 10-Year Monthly History + 2-Year Hourly Data)

## 1. Overview

This document enhances the previously shared methodology by leveraging a longer monthly history (10 years) together with two years of hourly 220 kV data. The goal is to produce robust, energy-consistent daily and hourly load curves for future years (e.g., 2030) using monthly consumption forecasts, while capturing seasonal patterns, day-type effects, and peak behavior.

## 2. Data Inventory

Required datasets:

• Monthly energy consumption: last 10 years (120 months)

• Hourly load data: last 2 years (for shape templates and growth elasticities)

• Calendar: weekdays/weekends/holidays/festivals

• (Optional) Weather: dry-bulb temperature, heat index, humidity, rainfall, irradiance

• (Optional) Exogenous drivers: industrial/commercial activity indices, population (for macro checks)

## 3. Modeling Strategy

### 3.1 Monthly Baseline (Trend + Seasonality from 10 Years)

Use the 10-year monthly series to estimate stable monthly seasonality and long-term trend. This improves the energy baseline and removes noise that might otherwise leak into hourly scaling. Options include: STL-like decomposition, seasonal regression with monthly dummies (and smooth trend), or state-space models. The result is a clean monthly baseline M_t (trend) and S_m (month seasonal indices).

Key outputs:

• Seasonal indices by calendar month (Jan..Dec)

• Long-term trend component for energy growth

• Residual noise for uncertainty bands

### 3.2 Seasonal and Day-Type Templates from 2-Year Hourly Data

Cluster historical days into seasons (e.g., Summer, Monsoon, Winter, Shoulder) and day types (weekday/weekend/holiday). For each (season, day-type), compute normalized per-unit hourly profiles. Also compute a peak-oriented template (e.g., 95th percentile profile) for peak safeguarding.

• Median hourly profile per (season, day-type)

• 95th percentile hourly profile per (season, day-type)

• Normalization: each template sums to 1.0 daily energy so it can scale cleanly

### 3.3 Monthly → Daily Allocation Using Day-Type Weights

Within each month, allocate total monthly energy to individual days using day-type weights derived from the 2-year history. Let w_d be the daily weight for day d (based on its (season, day-type) template aggregated to daily energy). Then:

D(d) = M_month × w_d / (Σ_{d∈month} w_d)

This preserves the monthly total while reflecting typical weekday/weekend/holiday energy differences. For holidays/festivals, apply special weights based on observed impacts (e.g., Diwali, Sankranti).

### 3.4 Daily → Hourly Allocation Using Normalized Hourly Templates

For each day, select the corresponding (season, day-type) hourly template H_tpl(h) normalized to sum to 1.0 over 24 hours. The base hourly allocation is:

H_base(h, d) = D(d) × H_tpl(h)

### 3.5 Relating Monthly Growth to Hourly Growth (Elasticity S(h))

Using overlapping months from the two years of hourly data, compute month-over-month energy growth ΔM and hour-specific growth ΔH(h). Define the elasticity:

S(h) = ΔH(h) / ΔM

Pool data across months and seasons to estimate robust S(h) curves, with regularization/shrinkage toward S(h)=1 when evidence is weak. Optionally model S(h) as a function of day-type and temperature bins to capture peak-sensitive hours (e.g., late afternoon summer).

### 3.6 Peak Correction and Extreme-Day Uplift

To reduce peak underestimation, blend the median and 95th percentile templates via a peak-adjustment factor:

H_adj(h, d) = H_base(h, d) × [1 + α·P(h)]

where P(h) is the hour-specific peak-sensitivity vector derived from historical peak days. Calibrate α (e.g., 0.05–0.15) to match observed monthly peak frequency and magnitude during backtests. Weather-normalized variants can use cooling/heating degree thresholds.

### 3.7 Energy Balance and Operational Constraints

• Energy preservation: Σ_h H_adj(h, d) over all days in the month equals M_month

• Non-negativity and realistic ramp-rate constraints between hours

• Optional transformer/substation coincidence factors if disaggregating spatially

• Cap daily peaks to historically plausible envelopes unless weather/exogenous factors justify exceedance

## 4. How the 10-Year Monthly History Improves Accuracy

• Stabilizes monthly seasonal indices, reducing noise in month-to-day allocations

• Improves long-term trend estimation and future monthly forecast plausibility checks

• Enables change-point detection (e.g., policy/tariff/COVID impacts) for regime-aware scaling

• Provides stronger priors for S(h): when ΔM is modest, constrain S(h) closer to 1; relax during structural shifts

## 5. Validation and Backtesting

Backtest on the two years with hourly data using only the monthly totals as inputs:

• Daily metrics: MAE/ MAPE of daily energy and peak

• Hourly shape metrics: MAPE by hour-of-day; RMSE on normalized profiles

• Peak accuracy: error on monthly system peak (magnitude and hour-of-occurrence)

• CVRMSE/NMBE for daily/hourly reconstructions; Reliability of top 10 peak days

## 6. Implementation Steps (Algorithm)

1) From 10-year monthly data, estimate monthly seasonal indices and trend.
2) Cluster two years of hourly data into (season, day-type) and compute normalized hourly templates (median and 95th).
3) For overlapping months, compute ΔM and ΔH(h); estimate S(h) with pooling/regularization.
4) For a target month, allocate M_month → daily using day-type weights (holiday adjustments applied).
5) For each day, allocate to hours using the matching hourly template: H_base(h, d) = D(d)×H_tpl(h).
6) Apply elasticity and peak corrections: H_adj(h,d) = H_base(h,d)×[1+α·P(h)] and any S(h) scaling if using elasticities directly on hours.
7) Rebalance to enforce exact monthly energy (proportional rescaling if needed).
8) Validate constraints (non-negativity, ramps) and export daily/hourly series.

## 7. Data Quality Checks

• Check monthly series for missing months and anomalies (extreme z-scores)

• Hourly gaps: fill via nearest-neighbor within cluster or template-based imputation

• Calendar alignment: correct timezone shifts and DST (if applicable)

• Holiday calendar verification for all 10 years

## 8. Deliverables

• (Season, day-type) normalized hourly template library (median and 95th)

• Day-type weight tables per month

• Elasticity vectors S(h) with confidence bands

• Reconstruction backtest report with metrics

• 2030 daily and hourly forecasts consistent with monthly inputs

## Appendix A. Notation & Formulas

ΔM = (M_y2 − M_y1) / M_y1 ;  ΔH(h) = (H_y2(h) − H_y1(h)) / H_y1(h)

S(h) = ΔH(h) / ΔM (hourly elasticity vs monthly energy)

Monthly → Daily:  D(d) = M_month × w_d / Σ w_d

Daily → Hourly:  H_base(h,d) = D(d) × H_tpl(h)

Peak correction:  H_adj(h,d) = H_base(h,d) × [1 + α·P(h)]

Energy rebalance:  scale hours by a common factor so that Σ_h Σ_d H_adj = M_month

## Appendix B. Example Walkthrough (Hypothetical)

Assume June-2030 monthly forecast M_month = 1000 MWh. There are 22 weekdays, 8 weekend days. From history, weekday weight = 1.05, weekend = 0.85. Then Σw = 22×1.05 + 8×0.85 = 31.9. For a weekday, D(d)=1000×1.05/31.9=32.9 MWh; for a weekend, D(d)=26.6 MWh. If the Summer-weekday template has an evening peak, allocate D(d) using H_tpl(h). If α=0.10 and P(h) peaks at hours 19–21, those hours get a 10% uplift before monthly rebalance.